WALL MOUNT LIGHT

INSTALLATION INSTRUCTIONS

MODEL NO 7972-16 AND 7972-23

Congratulations on your purchase of a Newport Coastal ‘Wall Light’.  You have purchased a safe, dependable, and easy-to-install security lighting system.  You can use this fixture at home for outdoor lighting applications.

WHAT’S INCLUDED:

A – Wall Light            B – Fixture Mounting Screws (2)            C – Swivel Mounting Bracket            D – Mounting Bracket Screws (2)            E – Wire Connectors (3)            F – Nut Caps (2)            G – Decorative Plastic Panels (4)         			                                        Tools Required:  Phillips Screwdriver, Silicon Sealant or Caulk

IMPORTANT SAFETY INFORMATION

THIS PRODUCT MUST BE INSTALLED IN ACCORDANCE WITH ALL APPLICABLE INSTALLATION CODES.

- IF YOU ARE NOT FAMILIAR WITH ELECTRICAL WIRING, IT IS RECOMMENDED THAT YOU SECURE THE SERVICES OF A WELL QUALIFIED ELECTRICIAN WHO CAN ASSURE THAT THIS INSTALLATION MEETS LOCAL, STATE, AND NATIONAL CODES.

- BE SURE POWER IS TURNED OFF AT THE MAIN POWER PANEL OR FUSE BOX PRIOR TO INSTALLATION OR REPLACING FIXTURE.

- Before installing this fixture, be sure to check with local ordinances regarding approved outdoor lighting.  This fixture style may not be allowed in some areas.

- This fixture is for outdoor use only.

- This fixture should be mounted to a grounded junction box marked for use in wet locations.

- The electrical system and the method of connecting the fixture to the system must be in accordance with local, state, and national electric codes.

- This fixture is to be connected to a 120 volt, 60 Hz power source.  Connecting to a different power source may create a hazard and will void the warranty.

- This fixture is suitable for wet locations.

- Do not use near or around flammable/combustible material or liquids.

- When replacing the lamp (bulb), use only Type A medium base lamp (bulb) – 60 Watt Maximum.  Never use a lamp (bulb) with a higher wattage than the stated maximum allowed on the fixture.  Using a higher wattage lamp may cause a fire hazard and/or damage to the unit.

- The lamp (bulb) and fixture can get hot during use.  Turn off the power and allow the fixture to cool down for at least 10 minutes prior to working on the unit.

- Do not attempt to open the fixture.  There are no serviceable parts inside.

- CLEAN ONLY WITH MILD SOAP AND WATER.  AVOID SPRAYING WITH CLEANSERS, INSECTICIDES, AND PESTICIDES THAT ARE PETROLEUM BASED HYDROCARBON PRODUCTS.  THESE WILL DEGRADE LUMINAIRE.

PRIOR TO INSTALLATION

Carefully read through the entire installation instructions before installing this security light fixture.

WARNING:  BE SURE POWER IS OFF AT MAIN POWER PANEL/FUSE BOX BEFORE INSTALLING OR REPLACING FIXTURE.  THIS PRODUCT MUST BE INSTALLED IN ACCORDANCE WITH THE APPLICABLE INSTALLATION CODES BY A PERSON FAMILIAR WITH THE CONSTRUCTION AND OPERATION OF THE PRODUCT AND THE HAZARDS INVOLVED.

INSTALLATION INSTRUCTIONS

BE SURE POWER IS TURNED OFF AT THE MAIN POWER PANEL OR FUSE BOX PRIOR TO INSTALLATION OR REPLACING FIXTURE.

- Remove old lighting fixture.  If installing at a new location, contact a qualified electrician.

- Install Plastic Decorative Panels (G) beginning with the panel under the ‘roof’ hinge area, by sliding panel (G) down slot on inside of cage.  Make sure

panel (G) is pushed all the way down into cage slot.  Lift panel holding tab away from panel (G) edge, and then snap panel (G) into place under panel

holding tab.  Refer to Figure 1.  Repeat with remaining panels.  Note:  Insert with raised decorative design facing outward.

- Insert two (2) Fixture Mounting Screws (B) - as shown in Figures 2 and 3, through back of Swivel Mounting Bracket (C).

- Attach Swivel Mounting Bracket (C) onto the junction box using two (2) Mounting Bracket Screws (D).  Pull the supply wires from the junction box

under the Swivel Mounting Bracket (C) in preparation for wiring.

- WIRING – See Figure 4

- Connect the Black wire from Wall Light (A) to Black wire of the supply circuit using Wire Connectors (E).

- Connect the White wire from Wall Light (A) to White wire of the supply circuit using Wire Connectors (E).

- Connect the Ground wire from Wall Light (A) to Ground wire of the supply circuit using Wire Connectors (E).

- Insert wires into junction box.

- Attach Wall Light (A) to junction box by inserting two (2) Fixture Mounting Screws (B)

through mounting holes on Wall Light (A) and then secure using two (2) Nut Caps (F).

See Figure 3.  Tighten Nut Caps (F) clockwise.

- Install Type A Medium Base lamp (bulb – not included)) into socket housing.  Lamp wattage NOT to

exceed 60 Watts.  Refer to ‘Installing or Replacing Lamp’ Section.

- IMPORTANT:  Apply a weatherproof silicone caulk or sealant around the fixture base to reduce chance of water

leaking into the fixture body.

- Turn ON the power.

INSTALLING OR REPLACING LAMPS

- WARNING:  Turn off the power and allow the fixture to cool down for at least 10 minutes prior to working on the unit.

- When replacing the lamp (bulb), use only Type A Medium Base lamp (bulb) - 60 watts maximum, in lamp (bulb) socket.

- To remove existing lamp (bulb) gently twist lamp (bulb) counterclockwise until it becomes free of fixture.  Install new

lamp (bulb) by turning clockwise.  Do not over-tighten lamp (bulb).

TROUBLESHOOTING

Does not function at all.

●  Make sure wall switch (if applicable) is ON and that the wiring is correct.

- Check that the circuit breaker, fuse, and main circuit breaker or main power switch are ON.

- Check lamp (bulb).

LIMITED LIFETIME WARRANTY

This product is warranted to be free from defects in material and workmanship for the life of the product. If a defect in material or workmanship occurs, call 1-800-562-5625 for instructions on how to have the defective product repaired or replaced free of charge.

LIMITATIONS

• This warranty only covers Newport Coastal assembled lighting products and is not extended to other components or equipment that is used in conjunction with the product.

• This warranty is extended to original purchaser of the product. A purchase receipt or other approved proof of purchase is required before warranty service will be rendered.

• Unauthorized service or modification to this product or any furnished component will void this warranty in its entirety.

• THIS WARRANTY IS EXPRESSLY IN LIEU OF ALL OTHER WARRANTIES, EXPRESSED OR IMPLIED, INCLUDING ANY WARRANTY, REPRESENTATION OR CONDITION OF MERCHANTABILITY OR THAT THE PRODUCTS

ARE FIT FOR ANY PARTICULAR PURPOSE OR USE, AND SPECIFICALLY IN LIEU OF ALL SPECIAL, INDIRECT, INCIDENTAL, OR CONSEQUENTIAL DAMAGES (INCLUDING BUT NOT LIMITED TO ANY LOSS OF BUSINESS      OR PROFITS, WHETHER OR NOT FORESEEABLE) RESULTING FROM THE USE OF THE PRODUCT OR ARISING OUT OF ANY BREACH OF THIS WARRANTY.

Some states or provinces do not allow the exclusion of incidental or consequential damages, so the above limitation or exclusion may not apply.

• This warranty gives you specific rights, and you may also have other rights that vary from state to state.

NOT COVERED

• Failure of product as a result of an accident, misuse, abuse, negligence, alteration, or faulty installation, or any other failure not relating to faulty material or workmanship.

• Damage caused by separately purchased replacement bulbs.

• Corrosion and discoloration of brass components.

• Reimbursement for inconvenience, installation, setup time, loss of use, or unauthorized service.

Hampton Products International Corp                                                                                                                                       						Made in U.S.A.

50 Icon                                                                                                                                                                                     						© 2010 Hampton Products International Corp.

Foothill Ranch, CA  92610-3000                                                                                                                                            						999-60495  REVA  2/10